OWNERS MANUAL

MODEL:

2-DOOR WARMER/MERCHANDISER
Professional Food Warmer
PART NUMBER:

51040

115 volts, 60 hz


[image: image6.png]Be Careful
e



WARNING: Handling the cord on this product may expose you to lead, a chemical known to the State Of California to cause cancer and birth defects or other reproductive harm. WASH HANDS AFTER HANDLING. 
1. Electrical ground is required on this appliance.

2. Do not modify the power supply cord plug. If it does not fit the outlet, have a proper outlet installed by a qualified electrician.

3. Do not use an extension cord with this appliance.

4. Check with a qualified electrician if you are in doubt as to whether the appliance is properly grounded.

SAFETY INSTRUCTIONS
1.      Plug electrical cord into a certified electrical outlet with a corresponding voltage rating as listed on the machine model labels.

2.      Always unplug the electric cord when appliance is not in use and before cleaning, adjusting or maintaining this steamer. Do not operate with a damaged cord or plug or after the machine malfunctions, has been dropped or damaged. Use authorized service for examination, electrical or mechanical adjustment or repair.

3       This equipment will be HOT during use and for a short time after it is turned off.  BE CAREFUL. To avoid burns, do not let bare skin touch hot surfaces. If provided, use handles when moving this appliance. Keep combustible materials away from appliance.

4.      Extreme caution is necessary when any appliance is used by or near children and whenever the appliance is left operating and unattended.

5.      This appliance is not intended for use in wet locations. Never locate appliance where it may fall into a water container.

         Do not insert or allow foreign objects to enter any ventilation or exhaust opening as this may cause an electrical shock, fire or damage to the appliance.
[image: image2.jpg]



Benchmark USA Inc.  ●  25-J Brookfield Oaks Dr. ●  Greenville, SC 29607 ●  T: 864-312-5500  ●  F: 864-312-5505
[image: image3.jpg]



Assembly Instructions
1. The Warmer has an operator side (the side with the controls) and a customer side.

2. You will also note that there are six sets of shelf brackets on the operator side and three sets of shelf brackets on the customer side.

3. Remove the nylon packing ties on the shelves and brackets.

4. For Front Counter Applications you can position the shelves either in a flat or sloping format.
a. For Flat shelves – position the shelves using the three customer side brackets and the three corresponding height brackets on the operator side.

b. For Sloping shelves – position the shelves on the three customer side brackets and the higher brackets on the operator side.
5. For Rear Counter Applications you will want to position the shelves in a flat format.

6. Plug the machine into a properly grounded 120 volt outlet.
7. Turn on the warming element with the front switch (the machine will be on when the LED is illuminated).
8. Adjust the thermostat to the desired temperature.
9. Load the warmer with the desired amount of food product.
10. Turn the machine off when not in use.

Cleaning

1. Turn off the machine and unplug it from the power source.

2. Remove any remaining food product from the machine.

3. Remove the shelves from the Machine and clean in a sink or dishwasher with mild detergent.

4. Clean out any food product from the base of unit with a damp cloth and mild detergent.

5. Clean the glass panels and cover with any standards glass cleaner.

6. Re-assemble the machine.

7. Plug the machine back into the 120 volt power source.
[image: image1][image: image4.png]pwi switch

Description
On/Off switch
Themaostat
Themostat Dial
Heating Element
Heating Element mounting
Shelves
Shelf Bracket

Element Housing

Wiring Diagram

white E

Parts List

b
alament

LED

thermostat

Description
Glass Doors
Hinges
Handle
Magnet
otnke Plate
Canop
Glass Panel
LED

5408

5402 5407

5404 5405

14

To Order Parts Contact:

Benchmark USA,Inc.
25-] Brookfield Oaks Dr.

Greenville, 3C 29607 USA
Tel: 877-432-5500

NS A ™A™ rrear





[image: image5.png]



Three Year Warranty

THE WARRANTY:

Benchmark USA warrants the original purchaser of new food equipment manufactured by Benchmark USA to be free from defects in material or workmanship  from the date of the original purchase for a period of three years.  

WHO IS COVERED BY THIS WARRANTY:

The original purchaser who has provided proper proof of the original retail purchase and all other requested information.

WHAT IS WARRANTIED:

The obligation of Benchmark USA under this warranty is limited to the replacement of any parts or components that in the opinion of Benchmark USA are defective, F.O.B. the factory, or at any other location that Benchmark USA may designate.

WHAT VOIDS THE WARRANTY:

1.  Any piece of equipment that has been installed, operated or maintained inconsistently 

     with Benchmark USA operating instructions.

2.  Any  part  or  component  that has been modified, changed or altered from its original 

     configuration.

3.  Any   electrical  component   that   has  been   damaged  due  to  submergence  in  any   

     liquid.

4.  Any   part   or   component   that   has   been   subject   to  abuse,  misuse,  neglect  or 

     accidents.

EXTENT OF WARRANTY:

Any defective component, part or assembly returned to Benchmark USA will be replaced by Benchmark USA.  Benchmark USA will not be responsible for any expenses incurred by the customer under the terms of this warranty, nor shall it be responsible for any damages either consequential, special, contingent, or otherwise; or expenses or injury arising directly or indirectly from the use of a Benchmark USA product.  Any component, part or assembly returned to Benchmark USA must be returned at the customers expense along with proof of purchase.  Benchmark USA reserves the right to determine whether the terms of this warranty have been properly complied with.  In the event that the terms are not complied with, Benchmark USA shall be under no obligation to honor this warranty.    

Benchmark USA Inc.  ●  25-J Brookfield Oaks Dr. ●  Greenville, SC 29607 ●  T: 864-312-5500  ●  F: 864-312-5505












To Order Parts Contact:


Benchmark USA, Inc.


25-J Brookfield Oaks Dr.


Greenville, SC 29607 USA


Tel:  877-432-5500


Fax:  864-312-5505


Email:  parts@benchmarkusainc.com











�


WARNING ELECTRICAL SHOCK HAZARD.





FAILURE TO FOLLOW THESE INSTRUCTIONS COULD RESULT IN SERIOUS INJURY OR DEATH.








